# Lab 1 Material
Microcontroller Setup, First Firmware Upload, and Basic GPIO

Bachelor Degree Program in Informatics • Internet of Things Course

Purpose. This first laboratory meeting introduces students to the IoT lab environment, safe hardware handling, board programming, serial monitoring, and simple digital input/output through an LED and push button experiment.

# Lab Overview

| Item | Details |
| --- | --- |
| Meeting | Lab 1 |
| Estimated duration | 150–170 minutes |
| Main competencies | Set up the board, upload firmware, use Serial Monitor, control output pins, and read digital input |
| Suggested board | Arduino Uno / Nano, ESP32, or compatible development board |
| Main software | Arduino IDE or PlatformIO |
| Main outputs | Working basic circuit, tested program, observation sheet, and short lab report |

# Learning Objectives

• Identify the main parts of a microcontroller development board and explain the role of digital pins, power pins, and communication interfaces.

• Install and configure the development environment needed to upload firmware to a board.

• Write, upload, and test a basic program that blinks an LED and prints status messages through the serial interface.

• Read a push button as a digital input and use it to control an output action.

• Record simple experimental observations and explain common troubleshooting steps.

# Required Equipment and Software

| No. | Item | Qty. | Notes |
| --- | --- | --- | --- |
| 1 | Laptop/PC with Arduino IDE installed | 1 | Prepare USB driver if required |
| 2 | Microcontroller board (Arduino Uno / Nano / ESP32) | 1 | One board per group or per student |
| 3 | USB data cable | 1 | Use a data cable, not charge-only |
| 4 | Breadboard | 1 | Half-size is sufficient |
| 5 | LED | 1–2 | Any color |
| 6 | 220Ω or 330Ω resistor | 1–2 | For LED protection |
| 7 | Push button | 1 | Momentary tactile switch |
| 8 | Jumper wires | Several | Male–male recommended |

# Safety and Good Practice

| Before Powering the Board | While Working |
| --- | --- |
| Check wiring twice before connecting USB power. | Handle boards gently and avoid short circuits on the breadboard. |
| Use a current-limiting resistor with every external LED. | Save code versions clearly and label tested circuits. |
| Do not force connectors or insert components while powered if avoidable. | Disconnect power before changing complex wiring. |

# Pre-Lab Preparation

□ Install the Arduino IDE or confirm that the board package is already available in the lab computer.

□ Bring a USB cable that supports both data and power transfer.

□ Read the board pinout sheet distributed by the instructor.

□ Prepare a short answer: what is the difference between digital input and digital output?

# Lab Activities

## Activity A — Board Introduction and IDE Setup

– Identify the USB port, power LED, reset button, onboard LED, digital pins, analog pins, and GND/VCC pins.

– Open the development environment and select the correct board type and serial port.

– Compile and upload a simple starter program to verify that the toolchain works.

## Activity B — Serial Monitor Test

– Write a program that prints a welcome message, student/group name, and time counter to the Serial Monitor.

– Open Serial Monitor and confirm the correct baud rate.

– Observe what happens when the board is reset.

## Activity C — LED Blink Experiment

– Connect an LED and resistor to a digital output pin or use the onboard LED if needed.

– Blink the LED with two different timing values.

– Modify the program so the serial output states when the LED is ON and OFF.

## Activity D — Push Button Input

– Wire a push button as digital input using pull-up or pull-down logic.

– Display the button state in the Serial Monitor.

– Turn the LED ON only when the button is pressed.

## Activity E — Simple Challenge

– Create a small behavior: one press toggles LED state, or blinking speed changes when the button is pressed.

– Demonstrate the result to the instructor or lab assistant.

# Suggested Wiring

Option 1: Use the onboard LED on the default LED pin for the first blink test.

Option 2: External LED circuit: digital pin → resistor → LED anode, LED cathode → GND.

Option 3: Push button circuit: one side to digital input pin, the other side to VCC or GND depending on the chosen pull-up/pull-down configuration.

# Sample Starter Code Structure

// Adjust pins for your board const int ledPin = 13; // or built-in LED pin const int buttonPin = 2; void setup() { pinMode(ledPin, OUTPUT); pinMode(buttonPin, INPUT_PULLUP); // example choice Serial.begin(9600); } void loop() { int buttonState = digitalRead(buttonPin); digitalWrite(ledPin, buttonState == LOW ? HIGH : LOW); Serial.println(buttonState); delay(100); }

# Observation Worksheet

| Activity | What was changed? | Observed result | Troubleshooting notes |
| --- | --- | --- | --- |
| A |  |  |  |
| B |  |  |  |
| C |  |  |  |
| D |  |  |  |
| E |  |  |  |

# Questions for Discussion

1. Why does an LED usually require a resistor in series?

2. What is the purpose of Serial.begin() and why must the baud rate match the Serial Monitor setting?

3. What is the difference between INPUT, OUTPUT, and INPUT_PULLUP modes?

4. If the board is powered but the program cannot upload, what are three possible causes?

5. How would you modify the program so the LED toggles once for every button press instead of simply following the button state?

# Assessment Rubric

| Criterion | Weight | Score | Indicators |
| --- | --- | --- | --- |
| Hardware setup | 20% |  | Correct and tidy circuit; safe wiring |
| Program functionality | 35% |  | Code uploads successfully and required behavior works |
| Testing and troubleshooting | 20% |  | Student can explain debugging steps and verify outputs |
| Worksheet completeness | 10% |  | Observations are recorded clearly |
| Discussion and explanation | 15% |  | Answers show understanding of GPIO and serial communication |

# Post-Lab Report Format

• Title and student/group identity

• Objective of the lab

• List of components and board used

• Short explanation of the circuit

• Program listing or key code screenshot

• Results and observations

• Answers to discussion questions

• Conclusion and issues encountered

# Appendix